flicktty : 80 columns vic screen driver for lunix-------------------------------------------------   uSAGE: SH0.EXE ! FLICKTTY [-0..7]   uSING 80 COLUMNS WITH A c64 HAS MANY DIFFICULTIES. sINCE WE HAVE THE vic RESOLUTION OF 320X200, WE ARE LIMITED TO 4 PIXELS WIDE CHARACTERSTO BE SHOWN IN THE 80 COLUMNS TEXT SCREEN. tHIS DIFFICULTY STILL EXISTS WITH THE 'FLICKTTY' BUT NOT THE MEMORY AND SPEED DISADVANTAGES, WHICHCONVENTIONAL 80-COLUMNS DRIVERS HAS.   cURRENT IMPLEMENTATION OF 'FLICKTTY' FOR LUNIX HAS THE THROUGHPUT OF5250 CHARACTERS PER SECOND. tHIS SPEED IS PRACTICAL CHARACTERISTIC SPEED.iT IS MEASURED AND CALCULATED CONCERNING THE BAD RASTER LINES AND SMOOTHSCROLL (ONE LINE UP PER ONE SCROLL). uSING A RATHER OPTIMIZED TRICKS CANMAKE IT POSSIBLE TO GO UP TO 8000 CPS THEORETICALLY. tHIS WOULD MAKE THECONNECTIONS AT 57600BPS FLAWLESS.   vic MEMORY REQUIREMENT OF 'FLICKTTY' IS ONLY 3328 BYTES. tWO KILOBYTESARE USED TO HOLD PARTIAL SCREEN DATA, AND ONE KILOBYTES HOLD THE FONTDEFINITIONS. oNE PAGE OF MEMORY IS FOR THE SPRITES. oNLY ONE SPRITE ISUSED FOR THE MOMENT, WHICH IS THE CURSOR. bUT THE DRIVER ROUTINE EATSSEVEN PAGES OF SYSTEM MEMORY, i WAS SO LAZY THAT i DIDN'T DEALLOCATE THEINITIALIZATION ROUTINES, WHICH ARE TAKE APPROXIMATELY TWO PAGES.   tHE IDEA BEHIND THE 'FLICKTTY' IS SIMPLE: iLLUSION. a RASTER INTERRUPTROUTINE SWITCHES AND SHIFTS THE TEXT SCREEN MEMORIES AT EACH SCREENREFRESH. tHIS MAKE TWO HALF SCREENS TO COMPLEMENT EACH OTHER BY USING A FEATURE OF OUR BRAINS AND crtS. iF YOUR crt HAS SLOW PHOSPHORUS AND IFYOU HAVE CHOSE LOW CONTRAST AND INTENSE COLOR WITH PROPER COLOR PATTERNS.yOUR BRAIN CANNOT SENSE THE FLICKKER. hERE THE COLORS MAKE IMPORTANTCONTRIBUTIONS TO THIS ILLUSION. oUR EYES HAVE MAINLY TWO GROUPS OF SENSORS.oNE GROUP SENSES ONLY THE BRIGHTNESS AND BRAIN USE THESE SENSORS FORSHAPE DETECTION. oTHER GROUP GIVES THE COLOR INFORMATION, AND THEY ARENOT AS FAST AS THE BRIGHTNESS SENSORS, ESPECIALLY IF YOU ARE LACK OFVITAMIN a.    tECHNIQUE OF 'FLICKTTY' IS SIMPLE EITHER. fOR THE luNIX IMPLEMENTATIONi USED $A300-$AFFF AREA. wITH DEFINING THE FIRST SCREEN AS THE ONE AT THE$A400-$A7E7 AND THE SECOND SCREEN AT $AC00-$AFE7. i INITALIZED THE SCREENSBY USING THESE INSTRUCTIONS:LDA$DD00AND#$FESTA$DD00 ; SELECTION OF $8000-$BFFF FOR vic DISPLAYLDA#$10STA$D011 ; 24 ROWSLDA#$9ASTA$D018 ; SCREEN 1LDA#$C8STA$D016 ; NO SHIFTaND AT EACH RASTER INTERRUPT i USED THESE INSTRUCTIONS TO SWAP THE SCREENS:LDA$D018EOR#$20STA$D018 ; DISPLAY OTHER SCREENLDA$D016EOR#$04STA$D016 ; SHIFT 4 PIXELS RIGHT OR LEFTnOW, TOGETHER WITH THE FONT, THE IDEA IS CAPABLE OF SHOWING 80 COLUMNSWIDE SCREEN. tHE FONT CONSISTS OF 128 CHARACTER DEFINITIONS. cHARACTERSARE DEFINED ONLY IN THE MOST SIGNIFICANT NYBLES (LEFT FOUR BITS OF A BYTE).  tO WRITE SOMETHING ON THE SCREEN WE INTERLEAVE THE TEXT PROPERLY. fOREXAMPLE, PRINTING 'HELLO WORLD' HAS THIS REPRESENTATION:ODD NUMBERED CHARACTERS GO TO THE FIRST SCREEN:$A400 : 08 0C 0F 17 12 04 20 20 : HLLWRD  AND EVEN NUMBERED ONES GO TO THE SECOND SCREEN:$AC00 : 05 0C 20 0F 0C 20 20 20 : EL OL    tHEN AT EACH 1/50 SECONDS ONE OF THE SCREENS IS DISPLAYED WITH PROPERSHIFTING:       ------------------1/50 : iH L L W R D     i         i                i       i                i       ------------------        ------------------2/50 : i E L   O L      i         i                i       i                i       ------------------ ......AND SO ON,   oF COURSE THIS SWAPPING CAUSE A FLICKER ON THE SCREEN WHICH IS SOMEHOWANNOYING. bUT IT CAN BE REDUCED BY SEVERAL WAYS:1. mOUNT A QUALITY SCREEN FILTER TO YOUR MONITOR.2. kEEP THE CONTRAST ADJUST OF YOUR MONITOR AS LOW AS POSSIBLE.3. kEEP THE COLOR INTENSITY AS HIGH AS POSSIBLE.4. aDJUST THE BRIGHTNESS INBETWEEN LOW AND MIDDLE.5. cHOOSE PROPER COLORS (YOU HAVE ONLY 8 CHOICES WITH 'FLICKTTY')6. wEAR SUNGLASSES (IN THAT CASE INCREASE THE CONTRAST ADJUST)7. tAKE NO MORE VITAMIN a ;-)   aS FOR THE USAGE AND IMPLEMENTATION OF 'FLICKTTY', i CAN SAY THAT ITIS SIMPLE AND LIMITED. iT'S SIMPLE BECAUSE IT ONLY DIRECTS ITS INPUT TO THE SCREEN. fOR EXAMPLE, THESE USAGES ARE POSSIBLE:SH0.EXE ! FLICKTTYCAT <UNIX-TEXT-FILE> ! UNIX2CBM ! FLICKTTYED ! FLICKTTY   tHE IMPLEMENTATION IS LIMITED BECAUSE THE SCREEN DRIVER MIMICS ONLY THETTY DEVICE. tHAT IS, NO MOVEMENT UP OR DOWN, NO COLOR CHANGES, AND NO MOREREVERSE CHARACTERS.    bUT IT HAS SOME SPECIAL CHARACTER CODES TO REFRESH THE COLOR RAM.fOR INSTANCE PRESSING CONTROL-1 REFRESHS IT.    uPON EXIT (WHICH IS THE SITUATION OF NO INPUT) IT RECOVERS THE OLDSCREEN.   cURRENT IMPLEMENTATION FOR luNIX HAS ALSO OTHER DRAWBACKS. fOR EXAMPLETHERE ARE SCREEN FLUSHES WHEN THERE IS A ACCESS TO DISK-DRIVE. aLSO IFAN APPLICATION HAS LONG ATOMIC PROCESSES, IT ALSO CAUSE FLUSHES. tHESCROLLER WITHIN THE 'FLICKTTY' HAS ALSO A LONG ATOMIC AREA. WHICH REDUCESTHE MULTITASKING A LITTLE BIT. bUT THIS IS NOT A BIG PROBLEM BECAUSE ITCAN SCROLL 95 LINES IN A SECOND. aND FINALLY IT GIVES YOU ONLY 24 ROWSOF TEXT. aLSO NOTE THAT WHEN YOU SCROLL LONG TEXTS YOU CANNOT FOLLOW IT.tHIS IS CAUSED FROM THE IDEA, NOT THE CODE. yOU SIMPLE SEE THE UNSCROLLEDPART FOR THE 1/50TH OF A SECOND.    mOST OF THESE DRAWBACKS CAN BE AVOIDED FOR LATER RELEASES OF luNIX.tHEY ARE CAUSED FROM THE RESOURCE CONFLICTS, THEREFORE THEY ARE EASY TOBE CORRECTED. bUT AS FOR THE LAST ONE, SIMPLY THERE IS NO SOLUTION FOR IT.aCTUALLY THERE IS ONE SIMILAR TO fli ROUTINE, BUT EATS 63% OF cpu TIME.   fOR THE COLOR PATTERNS YOU ONLY HAVE 8 DIFFERENT SELECTIONS. yOU CANSELECT ONE OF THEM LIKE THIS:SH0.EXE ! FLICKTTY -3INSTEAD OF 3 YOU CAN ENTER A NUMBER 0 TO 7, DEFAULT IS 0.THE NUMBERS HAS THESE COLOR PATTERNS:     NUMBER  FONT COLOR      BACKGROUND COLOR     ------- --------------- ----------------        0    YELLOW          LIGHT-BLUE        1    LIGHT-GREEN     LIGHT-BLUE        2    YELLOW          PURPLE        3    LIGHT-GREEN     PURPLE        4    YELLOW          GREEN        5    LIGHT-BLUE      BLUE        6    GREEN           BLUE        7    CYAN            BLUEnOTE THAT BORDER COLOR SAME AS THE BACKGROUND COLOR. fURTHERMORE, THE FONTCOLORS ARE NOT THE SAME AS i GIVE ABOVE, THEY ARE MIXED COLORS CAUSED BYTHE FLICKERING.    iN CONCLUSION IT CAN BE SAID THAT THE USE OF 'FLICKTTY' IS THE ONLYVIABLE SOLUTION TO 80 COLUMNS PROBLEM, FOR LIMITED MEMORY AND SLOW c64MACHINES.iLKER fICICILARFILKER@NEWTON.PHYSICS.METU.EDU.TRE068214@ORCA.CC.METU.EDU.TRHTTP://NEWTON.PHYSICS.METU.EDU.TR/~FILKER/CBM.HTML---