This file contains information on building the 64NET/2 cable for an Amiga.

*The Amiga cable is DIFFERENT to the PC cable, you have been warned*

*64NET/2 uses a _DIFFERENT_ cable to 64net v1*

(this is for improved reliability, as well as speed).

The 64NET/2 cable uses 12 pin-to-pin connections as follows:

Parallel Port     User Port

2                 B

3                 C

4                 D

5                 E

6                 F

7                 H

8                 J

9                 K

11                M

12                B

13                8

18                N